en

FIXSCREEN® MINIMAL M - Freestanding (IM 1F)

Renson NV, Maalbeekstraat 10, 8790 Waregem – Belgium

Tel. +32 (0)56 62 71 11, info@renson.be www.renson.eu

2023 01 01 - PM_Screens

Product characteristics

(text highlighted in red can be deleted depending on your choice)

Installation

This product is installed in front of the window with fully finished head box -> Surface-mounted.

'Freestanding' F (cannot be coupled) without window behind it.

Necessary securing for head box width > 3,600 mm, in order to avoid bending

The head box can be equiped with following options:

- With a hook-in profile; hooks into a recess at the back of the head box, for easy head box securing. Additional installation dimension for hook-in = + 5 mm

- With a finishing profile which garanties a slick and nice finish when head box is visible mounted

- Without hook-in profile

The side supporting end caps of the head box, supporting the roll-up mechanism are equipped with pins, which connect the head box with the side guiding channels.

Because of the adjustable pins the head box can easily be installed after the side guiding channels.

One side supporting end cap is equipped with the cable connector of the electrical connection.

During the installation of the fabric set (fabric tube + fabric + motor), the electrical connection is made easily via the motor connector.

Connect&Go-technology: facilitates quick and easy (dis)assembly of the fabric set.

The cable feed is located following the choice of installation.

You can disassemble the fabric set on the side of the removable profile of the head box; the left or right position is determined in this way (cable feed).

Head box

Composition: extruded aluminium.

Head box design: Square (rectangular).

Dimensions: 120 mm height and 110 mm depth.

Side guiding channels

Composition:

Are made of extruded aluminium profiles.

- 3-piece: u-profile allows for easy disassembly of the fabric set; without complete disassembly of the side guiding channel, without drilling or channelling, which guarantees water and wind proofing

- Every side guiding channel is equipped with an powder coated base plate under 0° or 5° and made of cast aluminium

- Inner rail:

- Powder coated, aluminium

- Smooth-technology: coextruded, wear-resistant top layer (= Smooth and quiet, long-term durable operation, without additional maintenance)

- Neoprene buffer zones (= Compensate for gusts of wind and contribute to constant fabric tension)

Installation :

Fixed directly onto the frame/construction and then clicked together without screws.

Side guiding channel option:

XS-side guiding channel: W 20 mm x D 55 mm, in 3 parts.

Fabric set

Composition:

- Fabric tube: Galvanised steel, with a recessed detail in which the removable UPVC insert clicks into place

(= Counteracts horizontal line formation by limiting the penetration of the fabric and facilitates (dis)assembly of the fabric)

- Narrowed conical fabric tube end pieces (= Compensating for the thicker ends of the rolled-up zipper and thus guaranteeing perfect roll-up of the screen)

Connect&Go-technology: (Dis)assembly of the fabric set is facilitated by means of the electrical connection: Connect&Go-technology.

Click&Safe-technology: The fabric set can be clicked into the head box without screws so that the installer has his hands free to safely complete the installation.

Fabric

All fabrics consist of a single piece of fabric, except when the height is greater than the fabric tube width.

Confection :

- Horizontally

- Fixscreen-technology: the vertical borders are equipped with a symmetrical zipper

(= This symmetrical shape ensures that the fabric absorbs the wind load well. As a result, the fabric is firmly affixed in both side guiding channels and remains windproof in any position)

- Welding zipper: High-frequency, always on the least visible side. (= Durable and aesthetic adhesion of the zippers to the fabric)

Applicable dimensions of fabrics, see brochure.

Type of fabric :

● Fibre glass fabric (semi-transparent):

- Fire resistance: M1 (NFP 92503), C-s3d0 (Euroclass EN 13501-1)

- Weight: ± 520-620 g/m², thickness: 0.53-0.80 mm

● Polyester fabric (semi-transparent - insect-proof):

- Weight: ± 240 g/m²

Weighted bottom bar

Composition:

- Bottom bar: Extruded aluminium, solidly weighted with bars in galvanised steel. (= Facilitates the smooth guiding system and tension of the fabric)

- Ballast: Steel bar. The bar is covered with PE foam to prevent contact between the aluminium and steel and to buffer noise

- Dimensions bottom bar: H 35 mm x D 40 mm (excl. sealing strip)

Finishing :

- Welded seam hidden in the bottom bar

- Fitted with UPVC end pieces. Available in 4 colours: black, white, grey and cream

- Fitted with a UPVC sealing strip to close off the sill. Available in 2 colours: black and grey.

- Easy (dis)assembly of the fabric by a hidden UPVC click profile

Colour

Aluminum profiles (head box, side guiding channels and bottom bar): Powder coated (60-80 µm) or anodised (20 µm) in the same colour as the external joinery.

Side supporting end caps (cast aluminium): Powder coated in the same colour as the profiles.

In the case of anodised profiles (head box, side guiding channels and bottom bar), the side supporting end caps are powder coated in the most suitable colour (60-80 µm).

Control

Electric:

- 230 V AC tubular motor, without emergency manual control

- 230 V AC tubular motor, with obstacle detection (Detecto) and continuous fabric stretch compensation

The motor connection is included in the sun protection category and is provided with a uv-resistant cable.

The electrical connection and all additional wiring are included in the electrical category.

Connect&Go-technology: electrical connection

Facilitates quick and easy (dis)assembly of the screen set.

When installing the fabric set in the head box, the motor connector moves via the straight guiding system of the corresponding side supporting end piece in the cable connector; both connector clips are fixed into place and form a perfect electrical connection inside the head box.

Renson Connect

Optional control with Renson smartphone application for iOS and Android, including product details and product-specific tips & tricks in the app.

Warranty and product information via QR code

10-year warranty on coatings on the aluminium profiles.

7-year warranty on electronic motors with obstacle detection (Detecto).

5-year warranty on the windproof-technology if 'Freestanding' F:

- Zipper remains in side guiding channel

- Optimal adhesion of zipper to fabric

5-year warranty on defects arising from normal domestic use and regular maintenance.

5-year warranty on motors.

Every individual sun protection product can be identified afterwards and linked to a warranty number or a unique screen ID.

This identification is done thanks to a QR code, which is placed in an easily accessible location (can be consulted without using extra tools) and can be easily scanned using a smartphone or tablet.

The essential product details (dimensions, colours, fabric type, motor type, etc.) can be consulted at any time.

Wind class

This product meets European standard EN13561.

Guaranteed up to 100 km/h in closed position (EN 13561:2004+A1:2008 conform wind class 3). The wind resistance depends on the screen dimensions and is available upon request.

Wind tunnel test report 'Von Karman Institute for Fluid Dynamics' (EAR1819): wind resistance guaranteed up to 200 km/h in closed position (tested for a screen of 1,000 mm x 1,000 mm).

Standards and certificates

This product is manufactured in accordance with: EN 13561 and conforms to and/or has been tested in accordance with these standards.

Declaration of performance: DoP-2016SC00006.

AluEco en VMRG Sun protection certificate.