3

RENSON LOGGIA® SLIDING PANELS

LOGGIAWOOD® PARO 0140 WITH FRAME LG.040 (PA-G-5.0700)

Renson NV, Maalbeekstraat 10, 8790 Waregem – Belgium

Tel. +32 (0)56 62 71 11, info@renson.be www.renson.eu

2022 04 28 Ascha WIP

Product characteristics

(text highlighted in red can be deleted depending on your choice)

Description

Loggiawood® Paro 0140 is a sliding panel made up of a combination of various materials used to manage heat entry and natural daylight.

The concept combines the typical characteristics of extruded aluminium - in a strong circumferential frame LG.040 with a surface treatment specified by the architect - and the naturalness of wood as panel infill.

The unique wooden blade fastening technique - without visible joints - allows for a smooth dilatation of the blades in the frame profile, without any vibration noise.

If requested an FSC or PEPC timber label (guarantee for sustainable logging) can be supplied.

Standards

Aluminium-Alloy Al Mg Si 0.5 (F25):

- Standard: EN AW-6063

- Curing: T66

Aluminium pre-treatment:

- Standard DIN 5002155

Strength calculations are based on the following standards:

- ENV 1999-1-1: Calculations of aluminium constructions

- NBN B-03-002-2: Wind load - Dynamic effects

- EN 1991-1-4: Wind load

Panel meets all CE requirements as stated in the declaration of performance DoP/RP/001 on the basis of harmonised technical specifications according to EN 13659:2004.

Surface treatment - aluminium

Natural colour anodised F1 (20 µm): pre-treated and anodized.

Polyester powder coating in RAL-colours (60-80 µm): aluminium profiles corrosion resistance pre-treated (DIN 5002155) to guarantee a firm powder coating after which thermally painted.

System execution

Frame:

- Vertical frame profiles LG.040.07, laterally provided with a finishing profile LG.040.08 for level view, made from extruded aluminium

● Dimensions of the frame profile:

- Width: 40 mm

- Depth: 40 mm

- Minimum inertia moments Iy = 94,248 mm4 (strong axis); Iz = 67,516 mm4, where a minimal material thickness of 2.0 mm

- Depending on the height of the sliding panel the horizontal frame profile (above and/or below) can measure 70 mm in height instead of 40 mm

● Horizontal frame profile LG.040.09:

- Depth: 40 mm

- Height: 40 mm

- Minimum inertia moments Iy = 75,158 mm4 (strong axis); Iz = 72,301 mm4, where a minimal material thickness of 2.0 mm

● Horizontal frame profile LG.040.70:

- Depth: 40 mm

- Height: 70 mm

- Minimum inertia moments Iy = 317,123 mm4 (strong axis); Iz = 116,033 mm4, where a minimal material thickness of 2.0 mm

- Invisible corner joints by means of finishing profile and end caps

Blades:

- Parallelogram shaped wooden blades WRC.PL.065

- High quality wooden blades of WRCedar Clear n°2 - untreated, suitable for natural aging

● Dimensions:

- Height: 16 mm

- Depth: 65 mm

- Critical angle for incoming sun rays: 47°

- Perpendicular visual opening: 29 mm

- Blade pitch: 90 mm (11 blades at 1 meter high)

- Blade angle: 57° (relative to the vertical direction)

- Minimum inertia moments Iy = 223.630 mm4 (strong axis); Iz = 15.126 mm4

- Blades incorporated discreetly and without visible fixings in the vertical side profiles of the frame

- The fastening technique allows for a smooth dilatation of the blades in the frame profile, without any vibration noise

Slide fittings:

- High quality maintenance free sliding fittings, comprise of:

- sliding rail (above) with trolley / suspension bracket and adjustable railstop or with rear rails

- guiding system (under) aluminium L-profile 20/20/4 and plastic guiding system

- lateral rubberstop to prevent damage to the panel

- Movement capabilities of the sliding panel (according to chosen design): simple - or symmetrical sliding

- Motorised control - according to chosen design

- 230 V motor - for connection to simple switch or buildings control system

- Slide fittings CE approved

System properties:

- Combination of sturdy aluminium frame and naturally durable wooden blades made of Western Red Cedar

- Frame profile with level view and invisible inner corner joints

- Depending on the height of the sliding panel the horizontal frame profile (above and/or below) can measure 70 mm in height instead of 40 mm

Dimensions:

- Maximum possible dimensions depend on local wind load qb

Optional accessories:

- Flexible under guiding system Flexguide®: self-regulating under guiding system for sloping areas with L-, T of U-shaped (incorporated) under guiding systems profile - see separate specification sheet